SECURE DATA LOGGER

+ Hardware Signing Module

A Complete Study Guide — From Concept to Working Firmware

| Platform | STM32F429I-DISC1 ¦ ARM Cortex-M4 ¦ HSI 16 MHz |
| --- | --- |
| Crypto | SHA-256 + HMAC-SHA256 — written from scratch, pure C, zero libraries |
| Storage | Internal Flash Sector 11 (0x080E0000, 128 KB) |
| Interface | USART1 PA9/PA10 9600 baud bare-register driver |
| Toolchain | Keil MDK v5 ¦ ARM Compiler v6 ¦ Python 3 PC tools |
| Key feature | Every sensor block is hashed, chained, and HMAC-signed. Tamper = instant fail. |

Embedded Systems Project  |  STM32F429I-DISC1

# Chapter 1: The Big Picture — What Are We Building?

Before touching any code, understand the core idea in one sentence:

Core Idea

A microcontroller that reads sensor data, stores it in a cryptographically

linked chain of blocks (like a mini-blockchain), signs each block with a secret

key, and can also sign arbitrary external messages on demand — just like a

hardware wallet or Hardware Security Module (HSM).

## 1.1 Why Is This Interesting?

Most embedded projects read a sensor and display it. That's it. This project adds three layers on top:

- Tamper evidence — if anyone edits stored data, verification fails immediately

- Authentication — every record is signed with a secret key on the device

- Signing oracle — the device can sign any message sent to it over UART, like a real HSM

## 1.2 The Data Flow in One Diagram

SENSOR (simulated)

|

v

SensorData { temp, pressure, humidity, timestamp }

|

v

block_to_hashable()  -- serialize 56 bytes

|

v

SHA-256(56 bytes)  -->  hash[32]

|

v

HMAC-SHA256(secret_key, hash)  -->  signature[32]

|

v

Block { index, ts, data, prev_hash, hash, sig, magic }

|

v

Flash Sector 11  (0x080E0000)

## 1.3 What Makes It Tamper-Evident?

Each block stores the hash of the PREVIOUS block inside itself (called prev_hash). So:

- Block 0: prev_hash = 00 00 00 ... 00  (zeros, genesis)

- Block 1: prev_hash = SHA256(Block 0 data)

- Block 2: prev_hash = SHA256(Block 1 data)

- Block N: prev_hash = SHA256(Block N-1 data)

If you edit Block 0's temperature value:

- Block 0's hash changes

- Block 1's prev_hash no longer matches Block 0's new hash

- Verification detects the mismatch at Block 1 and reports INVALID

- Every block after 0 also fails — the entire chain is broken from the tamper point

The HMAC signature adds a second layer: even if an attacker edits Block 0 AND recomputes its hash, they cannot forge the HMAC without the secret key that is stored only in device RAM.

# Chapter 2: Hardware — STM32F429I-DISC1

## 2.1 Board Overview

| Component | Details | Used In This Project |
| --- | --- | --- |
| MCU | STM32F429ZIT6, ARM Cortex-M4F | Everything |
| Clock | HSI 16 MHz internal RC oscillator | All bare-register code |
| Flash | 2 MB internal, 12 sectors | Code in sectors 0-10, chain in sector 11 |
| RAM | 256 KB | Stack, key, buffers |
| USART1 | PA9=TX, PA10=RX | UART CLI (9600 baud) |
| LED Green | PG13 | Blinks on auto-log |
| LED Red | PG14 | Fatal error indicator |
| ST-LINK | On-board SWD debugger | Flashing firmware via Keil |
| LCD | ILI9341 240x320 (SPI) | Not used — future expansion |

## 2.2 Why HSI Instead of HSE?

The Discovery board has an 8 MHz external crystal (HSE). Using HSE requires PLL configuration and adds boot complexity. HSI (internal 16 MHz RC oscillator) starts instantly with no external dependency. For this project, 16 MHz is sufficient. The UART baud rate divider (1667) is calculated for exactly 16 MHz, making the baud rate accurate.

## 2.3 Flash Memory Map

| Sector | Address | Size | Contents |
| --- | --- | --- | --- |
| 0-7 | 0x08000000 | 16 KB each | Interrupt vectors + firmware code |
| 8 | 0x08060000 | 64 KB | Firmware code (overflow) |
| 9-10 | 0x08080000 | 128 KB each | Unused — reserved |
| 11 | 0x080E0000 | 128 KB | BLOCK CHAIN STORAGE (our data) |

## 2.4 UART Physical Connection

The ST-LINK v2 on the Discovery board does NOT bridge UART to USB reliably on all board revisions. The working solution:

- USB-to-UART adapter (CP2102 / CH340 / FT232)

- Adapter TX  ->  PA10 (USART1 RX) on board

- Adapter RX  ->  PA9  (USART1 TX) on board

- Adapter GND ->  GND on board

- PuTTY: COM port = whatever adapter shows in Device Manager, 9600 baud, 8N1, no flow control

# Chapter 3: Project File Structure

All firmware lives in the Core/ folder. All PC tools are Python scripts.

SecureLogger/

Core/

sha256.h / sha256.c       -- SHA-256 from scratch (FIPS 180-4)

crypto.h / crypto.c       -- HMAC-SHA256 + key gen + sign/verify API

blockchain.h / blockchain.c -- Hash chain + bare-register Flash storage

sensor_sim.h / sensor_sim.c -- Simulated sensor data (replace with real sensor)

uart_hw.h / uart_hw.c     -- Bare-register USART1 driver

uart_cmd.h / uart_cmd.c   -- UART command line interface

main.c                    -- Entry point, init, polling loop

PC_Tools/

verifier.py               -- Reads dump over UART, verifies chain + HMACs

wallet_client.py          -- Interactive terminal + tamper/challenge demos

## 3.1 Dependency Order (Build Mental Model)

sha256.c

|

v

crypto.c   (uses sha256 for HMAC + key compression)

|

v

blockchain.c  (uses crypto for sign/verify, uses Flash registers directly)

|

sensor_sim.c  (standalone, no dependencies)

|

+-- uart_hw.c  (standalone bare-register UART)

|

v

uart_cmd.c  (uses blockchain + crypto + uart_hw)

|

v

main.c  (ties everything together)

# Chapter 4: SHA-256 — From Scratch

## 4.1 What SHA-256 Does

SHA-256 takes any input (any length) and produces a fixed 32-byte (256-bit) output called a hash or digest. Key properties:

- Deterministic: same input always gives same output

- One-way: you cannot reverse a hash to find the input

- Avalanche effect: changing 1 bit of input changes ~50% of output bits

- Collision resistant: finding two inputs with the same hash is computationally infeasible

## 4.2 The Context Structure

typedef struct {

uint32_t state[8];   // H0..H7: the running hash state

uint64_t bit_count;  // total bits processed so far

uint8_t  buf[64];    // partial block buffer (pending bytes)

uint32_t buf_len;    // how many bytes are in buf right now

} SHA256_CTX;

## 4.3 Initial Hash Values H0..H7

These are fixed constants defined by FIPS 180-4. They are the first 32 bits of the fractional parts of the square roots of the first 8 prime numbers (2, 3, 5, 7, 11, 13, 17, 19):

ctx->state[0] = 0x6a09e667;  // frac(sqrt(2))

ctx->state[1] = 0xbb67ae85;  // frac(sqrt(3))

ctx->state[2] = 0x3c6ef372;  // frac(sqrt(5))

ctx->state[3] = 0xa54ff53a;  // frac(sqrt(7))

ctx->state[4] = 0x510e527f;  // frac(sqrt(11))

ctx->state[5] = 0x9b05688c;  // frac(sqrt(13))

ctx->state[6] = 0x1f83d9ab;  // frac(sqrt(17))

ctx->state[7] = 0x5be0cd19;  // frac(sqrt(19))

## 4.4 The Round Constants K[0..63]

64 constants, each derived from the cube roots of the first 64 primes. They are what make SHA-256 unpredictable — without them, the algorithm would be invertible.

## 4.5 The Four Non-Linear Functions

These operate on 32-bit words and create the mixing and non-linearity:

// CH (Choose): if bit of e is 1, take f's bit; else take g's bit

#define CH(e,f,g)   (((e) & (f)) ^ (~(e) & (g)))

// MAJ (Majority): output 1 if at least 2 of {a,b,c} have a 1 bit

#define MAJ(a,b,c)  (((a)&(b)) ^ ((a)&(c)) ^ ((b)&(c)))

// Sigma0: rotational mix of working variable a

#define SIG0(x)  (ROTR(x,2) ^ ROTR(x,13) ^ ROTR(x,22))

// Sigma1: rotational mix of working variable e

#define SIG1(x)  (ROTR(x,6) ^ ROTR(x,11) ^ ROTR(x,25))

// sigma0/sigma1: used in message schedule expansion

#define sig0(x)  (ROTR(x,7)  ^ ROTR(x,18) ^ ((x)>>3))

#define sig1(x)  (ROTR(x,17) ^ ROTR(x,19) ^ ((x)>>10))

## 4.6 The Compression Function — 64 Rounds

For each 64-byte block of data, 64 rounds of mixing are applied to 8 working variables a..h:

// Build message schedule W[0..63]

for i = 0..15:  W[i] = big_endian_read(block[i*4..i*4+3])

for i = 16..63: W[i] = sig1(W[i-2]) + W[i-7] + sig0(W[i-15]) + W[i-16]

// Load working variables

a=state[0], b=state[1], c=state[2], d=state[3]

e=state[4], f=state[5], g=state[6], h=state[7]

// 64 rounds

for i = 0..63:

T1 = h + SIG1(e) + CH(e,f,g) + K[i] + W[i]

T2 = SIG0(a) + MAJ(a,b,c)

h=g; g=f; f=e; e=d+T1; d=c; c=b; b=a; a=T1+T2

// Add back to state

state[0]+=a; state[1]+=b; ... state[7]+=h

## 4.7 Padding

Before final hashing, the message is padded to a multiple of 512 bits:

- Append byte 0x80 immediately after the message

- Append zero bytes until message length = 56 mod 64

- Append the original message bit-length as a 64-bit big-endian integer

This means even an empty message produces 64 bytes of padded input, and SHA-256 always produces exactly 32 bytes output.

## 4.8 Usage in This Project

// One-shot (for most uses):

uint8_t hash[32];

sha256(data, len, hash);

// Streaming (for HMAC which chains two SHA-256 calls):

SHA256_CTX ctx;

sha256_init(&ctx);

sha256_update(&ctx, part1, len1);

sha256_update(&ctx, part2, len2);

sha256_final(&ctx, hash);

# Chapter 5: HMAC-SHA256 — Keyed Authentication

## 5.1 What HMAC Adds Over SHA-256

SHA-256 alone is not enough for authentication. Anyone can compute SHA-256(data). HMAC binds a secret key to the hash so that only someone who knows the key can produce or verify the MAC.

| Property | SHA-256 alone | HMAC-SHA256 |
| --- | --- | --- |
| Tamper detection | Yes (hash changes) | Yes (hash changes) |
| Authentication | No (anyone can hash) | Yes (need secret key) |
| Forgery resistance | No | Yes |
| Speed at 16 MHz | ~2ms | ~4ms (two SHA-256 calls) |

## 5.2 HMAC Formula (RFC 2104)

ipad = 0x36 repeated 64 times

opad = 0x5C repeated 64 times

If key > 64 bytes: key = SHA256(key)  // normalize

K' = key zero-padded to 64 bytes

HMAC(K, message) =

SHA256( (K' XOR opad) || SHA256( (K' XOR ipad) || message ) )

// Read as: inner hash = SHA256(key_ipad + message)

//          outer hash = SHA256(key_opad + inner_hash)

## 5.3 Why This Design Is Secure

- The inner hash binds the message to the key (K' XOR ipad)

- The outer hash prevents length-extension attacks that would affect SHA-256 alone

- Without the key, an attacker cannot compute HMAC(K, modified_message)

- The constant-time comparison in crypto_verify() prevents timing-based key recovery

## 5.4 Why We Chose HMAC Instead of ECDSA

Design Decision: ECDSA vs HMAC

Original plan: ECDSA P-256 signatures (like Bitcoin).

Problem discovered: ECC scalar multiplication on STM32F429 at 16 MHz = 30-60 seconds per signature.

Root cause: 256 point doublings, each needing a 256-bit modular inverse (~512 GCD iterations).

Solution: HMAC-SHA256 — two SHA-256 calls, under 5ms, fully from scratch.

Trade-off: HMAC is symmetric (verifier needs key); ECDSA is asymmetric (public key for anyone).

For this closed system where the PC tool is trusted: HMAC is the correct engineering choice.

This is what AWS IoT, TOTP (Google Authenticator), and JWT HS256 all use.

## 5.5 Entropy Collection for Key Generation

The secret key is generated from hardware entropy at boot using the ARM DWT (Data Watchpoint and Trace) cycle counter:

// Enable DWT cycle counter (counts CPU clock cycles)

CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;

DWT->CYCCNT = 0;

DWT->CTRL  |= DWT_CTRL_CYCCNTENA_Msk;

// Sample 64 bytes with variable-length busy waits

// The busy-wait length depends on DWT->CYCCNT, creating timing jitter

// Even small jitter at 16 MHz = millions of cycles variation per sample

for (int i = 0; i < 64; i++) {

volatile uint32_t t = DWT->CYCCNT;

for (volatile int j = 0; j < (int)(t & 0x7F) + 16; j++);

t ^= DWT->CYCCNT;  // XOR with new value for mixing

t ^= (t << 13); t ^= (t >> 7); t ^= (t << 17);  // Xorshift mix

raw[i] = (uint8_t)(t & 0xFF);

}

// Compress 64 noisy bytes into 32-byte key via SHA-256

sha256(raw, 64, g_key);

The key is stored in RAM only. It is regenerated each boot. This means verification requires the device (or a stored key export). For persistent keys, the key would be stored in Flash OTP (One-Time Programmable) region — a natural future extension.

# Chapter 6: The Hash Chain (Blockchain)

## 6.1 Block Structure

typedef struct {

uint32_t   index;                    // 4 bytes: block number (0, 1, 2...)

uint32_t   timestamp;                // 4 bytes: simulated seconds since boot

SensorData data;                     // 16 bytes: temp, pressure, humidity, ts

uint8_t    prev_hash[32];            // 32 bytes: SHA-256 of previous block

uint8_t    hash[32];                 // 32 bytes: SHA-256 of THIS block's fields

uint8_t    signature[32];            // 32 bytes: HMAC-SHA256(key, hash)

uint32_t   magic;                    // 4 bytes: 0x0B10CCA0 (sanity marker)

} Block;                                 // Total: 124 bytes per block

## 6.2 What Gets Hashed (The Hashable Fields)

Only specific fields are serialized for hashing. This is the exact byte order — the Python verifier must match this exactly:

static size_t block_to_hashable(const Block *b, uint8_t *buf) {

size_t pos = 0;

memcpy(buf+pos, &b->index,            4); pos += 4;  // index

memcpy(buf+pos, &b->timestamp,        4); pos += 4;  // block timestamp

memcpy(buf+pos, &b->data.temperature, 4); pos += 4;  // temp (0.01 C units)

memcpy(buf+pos, &b->data.pressure,    4); pos += 4;  // Pa

memcpy(buf+pos, &b->data.humidity,    4); pos += 4;  // humidity (0.01% units)

memcpy(buf+pos, &b->data.timestamp,   4); pos += 4;  // sensor timestamp

memcpy(buf+pos,  b->prev_hash,       32); pos += 32; // chain link

return pos;  // = 56 bytes total

}

// Then: sha256(buf, 56, b->hash)

// Then: hmac_sha256(key, hash, 32, b->signature)

## 6.3 Adding a Block — Step by Step

- Read sensor data (or simulate it)

- Fill block: index = block_count, copy sensor data, timestamp

- Set prev_hash: zeros if genesis block, else copy hash from last Flash block

- Serialize 56 bytes via block_to_hashable()

- SHA-256 the 56 bytes -> store in b.hash

- HMAC-SHA256(secret_key, b.hash) -> store in b.signature

- Write Block struct to Flash at address: 0x080E0000 + (index * 124)

- Increment block_count

## 6.4 Verification — Step by Step

- Load block from Flash (direct memory read — Flash is memory-mapped)

- Check magic == 0x0B10CCA0 and index == expected index

- Check prev_hash matches the hash stored in the previous block

- Reserialize the 56 bytes and recompute SHA-256 -> compare to stored hash

- Recompute HMAC-SHA256(key, hash) -> compare to stored signature

- If all pass: block OK. Update expected_prev = this block's hash. Repeat.

## 6.5 Error Codes from blockchain_verify_all()

| Return Value | Meaning |
| --- | --- |
| 0 | Chain valid — all blocks authentic |
| -(i*10 + 1) | Block i: magic number invalid (Flash corruption or uninitialized) |
| -(i*10 + 2) | Block i: index field wrong (struct layout issue) |
| -(i*10 + 3) | Block i: prev_hash does not match previous block's hash (chain broken) |
| -(i*10 + 4) | Block i: hash mismatch (data was modified after signing) |
| -(i*10 + 5) | Block i: HMAC signature invalid (data modified or wrong key) |

# Chapter 7: Bare-Register Flash Driver

We removed HAL Flash functions and wrote direct register access. This makes the code fully transparent — no hidden library behaviour.

## 7.1 Flash Unlock Sequence

STM32 Flash is locked by default to prevent accidental writes. The unlock sequence is a specific two-key write to FLASH->KEYR:

static void flash_unlock(void) {

if (FLASH->CR & FLASH_CR_LOCK) {   // only unlock if locked

FLASH->KEYR = 0x45670123UL;    // Key 1 (fixed by STM32 spec)

FLASH->KEYR = 0xCDEF89ABUL;    // Key 2 (fixed by STM32 spec)

}                                  // After this: FLASH->CR.LOCK = 0

}

static void flash_lock(void) {

FLASH->CR |= FLASH_CR_LOCK;        // Re-lock after writing

}

## 7.2 Sector Erase (Sector 11)

Before writing new blocks to a fresh address region, or when the user types 'erase', the entire sector is erased (all bytes set to 0xFF):

static void flash_erase_sector11(void) {

flash_wait_busy();           // wait if previous operation pending

FLASH->CR &= ~(FLASH_CR_SNB | FLASH_CR_PSIZE);  // clear sector bits

FLASH->CR |= (11 << FLASH_CR_SNB_Pos)   // select sector 11

| FLASH_CR_SER               // sector erase mode

| (2 << FLASH_CR_PSIZE_Pos); // 32-bit parallelism

FLASH->CR |= FLASH_CR_STRT;  // START the erase

flash_wait_busy();           // wait for completion (can take ~1 second)

FLASH->CR &= ~FLASH_CR_SER;  // clear SER bit

}

## 7.3 Word Programming

Flash is written 32 bits (4 bytes) at a time. Blocks are 124 bytes, so 31 word writes per block:

static void flash_write_word(uint32_t addr, uint32_t data) {

flash_wait_busy();

FLASH->CR &= ~FLASH_CR_PSIZE;

FLASH->CR |= (2 << FLASH_CR_PSIZE_Pos);  // 32-bit program width

FLASH->CR |= FLASH_CR_PG;                 // enable programming mode

*(__IO uint32_t *)addr = data;             // write to Flash address

flash_wait_busy();                         // wait for write to complete

FLASH->CR &= ~FLASH_CR_PG;                // disable programming mode

}

Note: Flash can only be written to an address that has been erased (contains 0xFFFFFFFF). Writing to an already-written address without erasing first produces corrupt data. This is why we track block_count — each new block goes to a fresh (0xFF) address.

# Chapter 8: Bare-Register UART Driver

We replaced HAL UART completely after discovering it was unreliable at HSI 16 MHz. The bare-register approach is simpler, faster, and gives full control.

## 8.1 USART1 Init

void uart_hw_init(void) {

RCC->AHB1ENR |= (1 << 0);    // Enable GPIOA clock

RCC->APB2ENR |= (1 << 4);    // Enable USART1 clock (APB2 peripheral bit 4)

// PA9 = TX: set to Alternate Function mode (binary 10)

GPIOA->MODER &= ~(3U << (9*2));    // clear bits 19:18

GPIOA->MODER |=  (2U << (9*2));    // set to AF mode

GPIOA->AFR[1] &= ~(0xFU << 4);     // clear AF bits for pin 9

GPIOA->AFR[1] |=  (7U   << 4);     // AF7 = USART1

// PA10 = RX: same AF7 setup

GPIOA->MODER &= ~(3U << (10*2));

GPIOA->MODER |=  (2U << (10*2));

GPIOA->AFR[1] &= ~(0xFU << 8);

GPIOA->AFR[1] |=  (7U   << 8);

// Baud rate: HSI=16MHz, APB2=16MHz, target=9600

// BRR = 16000000 / 9600 = 1666.67 -> rounded to 1667

USART1->BRR = 1667;

// Enable: UE=USART enable, TE=TX enable, RE=RX enable

USART1->CR1 = (1<<13) | (1<<3) | (1<<2);

}

## 8.2 Transmit and Receive

void uart_send_char(char c) {

while (!(USART1->SR & (1<<7)));  // Wait: TXE=1 means TX register empty

USART1->DR = (uint8_t)c;         // Load byte; hardware sends automatically

}

uint8_t uart_recv_char(void) {

while (!(USART1->SR & (1<<5)));  // Wait: RXNE=1 means byte received

return (uint8_t)(USART1->DR & 0xFF);

}

int uart_recv_ready(void) {

return (USART1->SR & (1<<5)) ? 1 : 0;  // Non-blocking check

}

## 8.3 Why Polling Instead of Interrupts

Interrupts would be slightly more efficient, but polling is simpler, more debuggable, and sufficient at 9600 baud. At 9600 baud, one byte takes ~1ms. The main loop runs much faster than that, so no bytes are missed.

# Chapter 9: Main Loop and Command Interface

## 9.1 Boot Sequence

int main(void) {

uart_hw_init();          // UART first — always works on HSI

// Print banner immediately so user knows board is alive

crypto_init();           // Collect entropy, generate key, compute pubkey

// Prints: 'Crypto OK' + 32-byte device identity

blockchain_init();       // Scan Flash sector 11, count existing blocks

// Prints: 'Blocks: N'

// LED init (PG13=green, PG14=red) via bare registers

// Print 'Ready. Type help'

// Enter polling loop

}

## 9.2 The Polling Loop

uint32_t inner = 0, outer = 0;

const uint32_t INNER_MAX = 160000UL;   // ~10ms per outer tick

const uint32_t OUTER_MAX = 12000UL;    // ~120 seconds total

while (1) {

// Priority 1: UART byte waiting?

if (uart_recv_ready()) {

uint8_t c = uart_recv_char();

uart_send_char((char)c);        // echo back to terminal

uart_cmd_process_char(c);       // feed to command handler

outer = inner = 0;              // reset auto-log timer

}

// Auto-log: every ~120 seconds when idle

if (++inner >= INNER_MAX) {

inner = 0;

if (++outer >= OUTER_MAX) {

outer = 0;

SensorData d = sensor_read();

blockchain_add(d);

GPIOG->ODR ^= (1<<13);  // toggle green LED

}

}

}

## 9.3 Command Handler — uart_cmd_process_char()

Called with each received byte. Accumulates bytes into rx_buf[]. On CR/LF, dispatches to the correct function:

| Command | Function Called | What It Does |
| --- | --- | --- |
| log | cmd_log() | sensor_read() + blockchain_add() + print result |
| verify | cmd_verify() | blockchain_verify_all() + print VALID or INVALID + error code |
| dump | cmd_dump() | Print all blocks: index, temp, hum, hash, prev_hash, HMAC |
| sign X | cmd_sign(X) | SHA-256(X) then HMAC(key, hash) — print both, the signing oracle |
| pubkey | cmd_pubkey() | Print SHA256(secret_key) — device identity commitment |
| status | cmd_status() | Print block count and chain validity |
| erase | cmd_erase() | Two-step confirmation then blockchain_erase() (sector 11 erase) |

# Chapter 10: Complete Final Code Files

All seven firmware files in their final working state. Read these alongside the chapter explanations above.

## 10.1 sha256.h

#ifndef SHA256_H

#define SHA256_H

#include <stdint.h>

#include <stddef.h>

typedef struct {

uint32_t state[8];

uint64_t bit_count;

uint8_t  buf[64];

uint32_t buf_len;

} SHA256_CTX;

void sha256_init   (SHA256_CTX *ctx);

void sha256_update (SHA256_CTX *ctx, const uint8_t *data, size_t len);

void sha256_final  (SHA256_CTX *ctx, uint8_t hash[32]);

void sha256        (const uint8_t *data, size_t len, uint8_t hash[32]);

#endif

## 10.2 crypto.h

#ifndef CRYPTO_H

#define CRYPTO_H

#include <stdint.h>

#include <stddef.h>

int  crypto_init(void);

void crypto_hash(const uint8_t *data, size_t len, uint8_t hash[32]);

int  crypto_sign(const uint8_t hash[32], uint8_t sig[32]);

int  crypto_verify(const uint8_t hash[32], const uint8_t sig[32]);

void crypto_get_pubkey(uint8_t pub_out[32]);

#endif

## 10.3 blockchain.h

#ifndef BLOCKCHAIN_H

#define BLOCKCHAIN_H

#include <stdint.h>

#include <stddef.h>

#include "sensor_sim.h"

#define BLOCK_SIG_LEN  32

#define MAX_BLOCKS     64

typedef struct {

uint32_t   index;

uint32_t   timestamp;

SensorData data;

uint8_t    prev_hash[32];

uint8_t    hash[32];

uint8_t    signature[BLOCK_SIG_LEN];

uint32_t   magic;

} Block;

void  blockchain_init(void);

int   blockchain_add(SensorData data);

int   blockchain_verify_all(void);

int   blockchain_count(void);

Block blockchain_get(int index);

void  blockchain_erase(void);

#endif

## 10.4 sensor_sim.h + sensor_sim.c

/* sensor_sim.h */

#ifndef SENSOR_SIM_H

#define SENSOR_SIM_H

#include <stdint.h>

typedef struct {

uint32_t temperature;  /* 0.01 C: e.g. 2547 = 25.47 C */

uint32_t pressure;     /* Pa: e.g. 101325              */

uint32_t humidity;     /* 0.01%: e.g. 6000 = 60.00%   */

uint32_t timestamp;    /* simulated seconds since boot  */

} SensorData;

SensorData sensor_read(void);

#endif

/* sensor_sim.c */

#include "sensor_sim.h"

#include "stm32f4xx.h"

static uint32_t tick = 0;

SensorData sensor_read(void) {

tick++;

SensorData d;

d.temperature = 2000 + (tick * 7) % 1500 + (tick*31 + DWT->CYCCNT) % 50;

d.pressure    = 101000 + (tick * 13) % 1000;

d.humidity    = 4500 + (tick * 11) % 3000;

d.timestamp   = tick * 5;

return d;

}

/* To use a real sensor: replace sensor_read() body with I2C/SPI reads.

Keep the SensorData struct exactly as-is.  Everything else unchanged. */

## 10.5 uart_hw.h + uart_hw.c

/* uart_hw.h */

#ifndef UART_HW_H

#define UART_HW_H

#include <stdint.h>

#include <stddef.h>

void    uart_hw_init(void);

void    uart_send_char(char c);

void    uart_send_string(const char *s);

void    uart_send_bytes(const uint8_t *data, size_t len);

void    uart_send_hex(const uint8_t *data, size_t len);

uint8_t uart_recv_char(void);

int     uart_recv_ready(void);

#endif

/* uart_hw.c  (key parts) */

void uart_hw_init(void) {

RCC->AHB1ENR |= (1<<0);          // GPIOA clock

RCC->APB2ENR |= (1<<4);          // USART1 clock

GPIOA->MODER &= ~(3U<<(9*2));  GPIOA->MODER |= (2U<<(9*2)); // PA9 AF

GPIOA->AFR[1] &= ~(0xFU<<4);   GPIOA->AFR[1] |= (7U<<4);   // AF7

GPIOA->MODER &= ~(3U<<(10*2)); GPIOA->MODER |= (2U<<(10*2));// PA10 AF

GPIOA->AFR[1] &= ~(0xFU<<8);   GPIOA->AFR[1] |= (7U<<8);   // AF7

USART1->BRR = 1667;              // 16MHz / 9600 baud

USART1->CR1 = (1<<13)|(1<<3)|(1<<2); // UE + TE + RE

}

void uart_send_hex(const uint8_t *data, size_t len) {

const char h[] = "0123456789ABCDEF";

for (size_t i=0; i<len; i++) {

uart_send_char(h[(data[i]>>4)&0xF]);

uart_send_char(h[data[i]&0xF]);

}

uart_send_string("\r\n");

}

## 10.6 main.c

#include "stm32f4xx.h"

#include "uart_hw.h"

#include "crypto.h"

#include "blockchain.h"

#include "uart_cmd.h"

#include "sensor_sim.h"

#include <string.h>

#include <stdio.h>

int main(void) {

uart_hw_init();

uart_send_string("Secure Data Logger + Wallet\r\n");

uart_send_string("SHA-256 + HMAC-SHA256 from scratch\r\n");

uart_send_string("Initializing crypto...\r\n");

if (crypto_init() != 0) {

uart_send_string("FATAL: crypto_init failed\r\n");

while (1);

}

uart_send_string("Crypto OK\r\n");

uint8_t pub[32];

crypto_get_pubkey(pub);

uart_send_string("DeviceID: ");

uart_send_hex(pub, 32);

blockchain_init();

char buf[48];

snprintf(buf, sizeof(buf), "Blocks: %d\r\n", blockchain_count());

uart_send_string(buf);

/* LED init: PG13=green, PG14=red */

RCC->AHB1ENR |= (1<<6);

GPIOG->MODER &= ~((3<<(13*2))|(3<<(14*2)));

GPIOG->MODER |=  ((1<<(13*2))|(1<<(14*2)));

uart_send_string("\r\nReady. Type 'help'\r\n> ");

uint32_t inner = 0, outer = 0;

while (1) {

if (uart_recv_ready()) {

uint8_t c = uart_recv_char();

uart_send_char((char)c);

uart_cmd_process_char(c);

inner = outer = 0;

}

if (++inner >= 160000UL) {

inner = 0;

if (++outer >= 12000UL) {

outer = 0;

blockchain_add(sensor_read());

GPIOG->ODR ^= (1<<13);

}

}

}

}

# Chapter 11: PC Tools — Python

## 11.1 verifier.py — What It Does

- Connects to device over UART

- Sends 'pubkey' command, reads device identity (32 hex bytes)

- Sends 'dump' command, parses all blocks from the output

- For each block: recomputes SHA-256 and HMAC-SHA256, checks chain linkage

- Prints PASS/FAIL per block with specific failure reason

## 11.2 wallet_client.py — Interactive Terminal

- Simple interactive loop: reads command from keyboard, sends to device, prints response

- --tamper flag: runs a step-by-step tamper demonstration for presentation

- --challenge flag: PC sends random nonce, device signs it, PC verifies — proves device identity

## 11.3 Running the Tools

# Install dependencies

pip install pyserial cryptography

# Edit PORT at top of each file:

PORT = "COM5"        # Windows

PORT = "/dev/ttyUSB0" # Linux/Mac

# Run verifier

python verifier.py

# Run interactive terminal

python wallet_client.py

# Run tamper demo (for presentation)

python wallet_client.py --tamper

# Run challenge-response demo

python wallet_client.py --challenge

# Chapter 12: Keil Build Guide — Step by Step

## 12.1 Project Setup

- Open Keil MDK v5

- Project -> New Project -> name it 'SecureLogger'

- Device: STM32F429ZITx

- Manage Run-Time Environment: tick CMSIS->CORE, Device->Startup, Device->HAL->Common/CORTEX/FLASH/GPIO/RCC/UART

## 12.2 Adding Source Files

Right-click Source Group 1 -> Add Existing Files. Add all of these:

- main.c

- sha256.c

- crypto.c

- blockchain.c

- sensor_sim.c

- uart_hw.c

- uart_cmd.c

## 12.3 Include Path

Project -> Options for Target -> C/C++ tab -> Include Paths: add the Core/ folder path.

## 12.4 Build and Flash

- Press F7 to build. Should show 0 Errors, 0 Warnings.

- Press F8 (or Flash -> Download) to program the device.

- Open PuTTY: Session -> Serial -> COM5 (your adapter port) -> 9600 -> Open.

- Press the black RESET button on the Discovery board.

- You should see the boot banner and 'Ready. Type help' within 1 second.

## 12.5 Common Errors and Fixes

| Error | Cause | Fix |
| --- | --- | --- |
| L6218E: Undefined symbol huart3 | uart_cmd.c compiled, main.c not in project | Add main.c to Source Group |
| L6200E: Symbol multiply defined | Test code accidentally left in uart_cmd.c | Restore uart_cmd.c to original |
| Nothing on PuTTY after reset | Wrong baud rate (was 9600, opened at 115200) | Use 9600 baud in PuTTY |
| Only 'H' shows on PuTTY | HAL_Delay() hanging (SysTick issue with HSI) | Use bare-register driver (no HAL) |
| log takes forever (>10s) | ECC scalar_mul too slow at 16 MHz | Use HMAC-SHA256 (already done) |
| Blocks not persisting on reset | Flash write failed (HAL timeout issue) | Use bare-register Flash driver |

# Chapter 13: Demo Script — For Presentation

Use this script when showing the project. Each step has a clear visual output on PuTTY that evaluators can see.

## Step 1 — Boot (30 seconds)

Power on / press reset. Show the banner printing live:

=====================================

Secure Data Logger + Wallet

SHA-256 + HMAC-SHA256 from scratch

=====================================

Initializing crypto...

Crypto OK

DeviceID: A3F2D18B...C47E91

Blocks: 0

Ready. Type 'help'

>

Say: 'At boot, the device generates a unique secret key from hardware timing noise, then derives a public device identity as SHA-256 of that key.'

## Step 2 — Log Blocks (1 minute)

Type 'log' five times. Show different simulated sensor values each time:

> log

Block #0 | Temp:20.00 C | Hum:45.00% | P:101000 Pa

> log

Block #1 | Temp:20.07 C | Hum:45.11% | P:101013 Pa

Say: 'Each log command reads sensor data, SHA-256 hashes it linked to the previous block, HMAC-signs the hash, and writes everything to Flash. The chain grows one block at a time.'

## Step 3 — Verify Chain

> verify

Verifying...

VALID - all blocks authentic

Say: 'Verification re-hashes every block, checks chain linkage, and re-computes the HMAC. All pass. The data is authentic.'

## Step 4 — Show a Block (dump)

> dump

[Block 1] ts=10 Temp=20.07 C Hum=45.11%

Hash: 3C9A7F12...D40E8811

Prev: A1B2C3D4...F0E1D2C3   <-- exactly Block 0's hash

HMAC: 2B48E301...C9F17A55

Say: 'Notice Block 1 Prev field is identical to Block 0 Hash. This is the chain. Edit any byte of Block 0, and this match breaks.'

## Step 5 — Wallet Feature (sign)

> sign Hello Evaluator

Msg:  Hello Evaluator

Hash: 8F14E45F...CCEFF923

HMAC: D3A1B029...7C4E8812

SIGNED OK

Say: 'This is the hardware wallet feature. The device SHA-256 hashes any message, then HMAC-signs it. No one can forge this signature without the device's secret key. This is how HSMs and hardware wallets work.'

## Step 6 — Tamper Demo (run verifier.py --tamper)

On PC, run: python wallet_client.py --tamper

The demo shows: verify passes, then simulates a byte flip in Block 0, then re-verifies and shows CHAIN INVALID at Block 0, cascading to all subsequent blocks.

Say: 'This is the critical proof. Any modification to any stored block is immediately detected. The attacker cannot re-sign without the key.'

# Chapter 14: Questions You Will Be Asked + Answers

## Q: Why HMAC and not ECDSA?

Answer

ECC scalar multiplication on STM32F429 at 16 MHz takes 30-60 seconds per signature.

This is because each of the 256 point doublings requires a 256-bit modular inverse,

which runs a binary GCD over 512 iterations of 256-bit arithmetic.

HMAC-SHA256 requires only two SHA-256 calls — under 5ms at 16 MHz.

For a closed system where the verifier is a trusted PC tool, HMAC provides

equivalent tamper-detection guarantees. This is what JWT HS256, TOTP,

and AWS IoT device authentication all use.

## Q: Is HMAC really cryptographically secure?

Answer

Yes. HMAC-SHA256 is standardized in RFC 2104 and FIPS 198-1.

It is provably secure under the assumption that SHA-256 is a pseudorandom function.

It is used in TLS 1.2, SSH, S/MIME, IPsec, and practically every security protocol.

An attacker without the 256-bit key cannot forge an HMAC even with unlimited queries.

## Q: Where is the secret key stored?

Answer

In RAM only. Generated fresh from DWT entropy at each boot.

This means after power cycling, old block signatures cannot be verified by the same

device without storing the key. For a production system, the key would be stored in

the Flash OTP (One-Time Programmable) region or a dedicated secure element.

For this project, the key-per-boot model demonstrates the concept correctly.

## Q: Why bare-register instead of HAL for UART?

Answer

HAL_UART_Transmit() uses HAL_GetTick() for timeout calculation.

HAL_GetTick() depends on SysTick being configured correctly for the clock speed.

With HSI at 16 MHz and no SystemClock_Config(), SysTick ticks at the wrong rate,

causing timeouts to expire immediately — transmitting only the first byte.

Bare-register UART waits directly on the TXE hardware flag, with no timing dependency.

This is more reliable, faster, and fully transparent.

## Q: What happens if Flash sector 11 fills up (64 blocks)?

Answer

blockchain_add() returns -1 and prints an error. No data loss occurs.

The user must type 'erase' (twice for confirmation) to erase sector 11 and start fresh.

With 64 blocks at one per minute auto-logging, the sector lasts over an hour.

For longer retention: use a larger sector, multiple sectors, or an SD card.

## Q: How does this compare to a real hardware wallet?

Answer

A hardware wallet (Ledger, Trezor) stores a private key and signs transactions.

This device stores a secret key and signs data and messages on demand.

The core function is identical: secure key storage + signing oracle.

Differences: we use HMAC (symmetric) vs real wallets using ECDSA (asymmetric);

real wallets use a secure element chip; we use MCU Flash.

The architecture, threat model, and signing flow are the same.

# Chapter 15: Real-World Applications

| Application | How This Project Maps To It |
| --- | --- |
| Cold chain / pharma logging | Signed temperature records prove vaccines were stored correctly. Tamper = FDA violation caught. |
| Industrial safety logs | Pressure/temperature event logs that cannot be backdated for insurance/legal purposes. |
| IoT device attestation | 'sign <nonce>' is exactly how AWS IoT and Azure IoT Hub prove device identity to cloud. |
| Electronic voting audit trail | Chained signed log of votes that cannot be modified without breaking the chain. |
| Automotive black box (EDR) | Crash data signed by vehicle ECU; chain proves data was not altered before insurance inspection. |
| Embedded HSM | Any system needing a trusted signing oracle — key never leaves device, all signing done on-chip. |
| Secure firmware update log | Log of every firmware update, signed and chained, proving no unauthorized update occurred. |

# Chapter 16: What to Add Next

| Feature | Effort | How to Add |
| --- | --- | --- |
| Real BME280 sensor (I2C) | Low | Replace sensor_read() with HAL_I2C_Master_Transmit/Receive. Keep SensorData struct. |
| ILI9341 LCD display | Medium | Board has it built-in. SPI driver + draw block count, chain status, device ID on screen. |
| PIN access control | Low | sha256(PIN) stored in Flash option bytes. Check at boot before allowing sign/log. |
| Persistent key across reboots | Medium | Store key in Flash OTP region (write-once). Load at boot instead of generating. |
| JSON block export | Low | Format dump output as JSON. Python verifier parses JSON directly — cleaner pipeline. |
| ECDSA (at full 168 MHz + DSP) | High | Run at full PLL 168 MHz. Use hardware multiply (DSP instructions) for bn_mod_mul. |
| Multiple sensor types | Low | Extend SensorData struct. Add GPS, accelerometer, voltage fields. |
| BLE/UART wireless | High | Replace wired UART with BLE module (HC-08). Same command protocol, wireless demo. |

# Quick Reference Card

| Item | Value |
| --- | --- |
| Board | STM32F429I-DISC1 |
| MCU | STM32F429ZIT6, Cortex-M4F |
| Clock | HSI 16 MHz (internal, no PLL) |
| UART | USART1, PA9=TX, PA10=RX, 9600 8N1 |
| Flash chain address | 0x080E0000 (Sector 11, 128 KB) |
| Block size | 124 bytes (max 64 blocks) |
| Hash algorithm | SHA-256, FIPS 180-4, from scratch |
| MAC algorithm | HMAC-SHA256, RFC 2104, from scratch |
| Key size | 256 bits (32 bytes), RAM only |
| Signature size | 256 bits (32 bytes) HMAC output |
| Hash chain field | 56 bytes serialized per block |
| PuTTY settings | 9600 baud, 8 data, 1 stop, no parity, no flow |
| Python packages | pip install pyserial cryptography |
| Keil compiler | ARM Compiler v6 (ARMCLANG) |